Name Tadesse Agegnehu

Id BDU1602496

# Assignment-2: PC Brand Evaluation – Compaq

## 1. Hardware Quality & Performance

| Category | Evaluation |
| --- | --- |
| CPU Performance | Compaq PCs usually use Intel Core i3/i5 (older 8th–11th Gen) or AMD Ryzen 3/5 processors. Suitable for office work, browsing, and light multitasking, not high-end workloads. Power efficiency is average; thermal output is manageable but not optimized for heavy loads. |
| GPU Performance | Mostly integrated GPUs (Intel UHD / Iris Xe or AMD Vega). Not designed for gaming or workstation tasks. Very limited graphics performance. |
| RAM Type & Speed | Typically DDR4 RAM, 8GB common. Some models allow RAM upgrades, but many budget laptops have one soldered slot. DDR5 is rare. |
| Storage | Uses SSD (SATA or PCIe Gen 3 NVMe) in newer models. Older models may include HDDs. Storage upgrade is usually possible. |

## 2. Thermals & Noise

| Category | Evaluation |
| --- | --- |
| Cooling System Quality | Basic cooling system. Designed for low-power CPUs. Not suitable for sustained heavy workloads. |
| Fan Noise Under Load | Quiet during light tasks. Fan becomes noticeable under multitasking or long usage. |
| Thermal Throttling | Can occur under continuous load due to basic thermal design. Acceptable for everyday tasks. |

## 3. Build Materials & Durability

| Category | Evaluation |
| --- | --- |
| Chassis Material | Mostly plastic chassis to reduce cost. |
| Hinges | Standard hinges; not reinforced like business-grade laptops. |
| Durability Certifications | No MIL-STD certifications. Not designed for rugged use. |

## 4. Design & Ergonomics

| Category | Evaluation |
| --- | --- |
| Weight & Portability | Lightweight to moderate. Suitable for students and office users. |
| Keyboard Quality | Average keyboard. Decent typing experience but limited key travel. Backlight may be absent in budget models. |
| Trackpad | Functional but basic. Adequate for normal use. |
| Hinge Design | Traditional clamshell design (no 360° or detachable). |
| Aesthetics | Simple, minimal, budget-oriented design philosophy. |

## 5. Display Quality

| Category | Evaluation |
| --- | --- |
| Resolution | Mostly Full HD (1920×1080). Older models may use HD panels. |
| Panel Type | IPS in newer models; TN panels in older or very low-cost models. |
| Brightness | Around 220–300 nits (average). |
| Color Coverage | ~60–65% sRGB. Not suitable for professional color work. |
| Refresh Rate | Standard 60 Hz. |
| Contrast Ratio | Average contrast. |
| Touchscreen | Rare; available only in selected models. |

## 6. Reliability & Brand Reputation

| Category | Evaluation |
| --- | --- |
| Reliability Track Record | Decent for entry-level systems. Not premium or long-term heavy-use machines. |
| User Reviews | Mixed reviews: praised for affordability, criticized for build quality. |
| Repairability Scores | Moderate repairability. Easier than ultra-thin laptops but not excellent. |
| Long-Term Performance | Stable for basic use; performance degrades faster under heavy workloads. |

## 7. Battery & Power Efficiency

| Category | Evaluation |
| --- | --- |
| Battery Life | 5–7 hours for normal usage. |
| Fast Charging | Limited or not available in most models. |
| Battery Health Features | Basic battery management via BIOS/Windows. |
| Power Adapter | Compact and lightweight charger. |

## 8. Software & Ecosystem

| Category | Evaluation |
| --- | --- |
| Preinstalled Bloatware | Some HP/Compaq utilities preinstalled; removable. |
| OS Options | Mostly Windows 10/11. Linux support is unofficial but possible. |
| Brand Software | Basic HP utilities (system diagnostics, updates). |
| Driver Stability | Stable drivers, but updates are less frequent than premium brands. |

## 9. Security Features

| Category | Evaluation |
| --- | --- |
| BIOS/UEFI Security | Standard BIOS security features. |
| TPM | TPM 2.0 available in newer models. |
| Fingerprint Reader | Rare in budget Compaq models. |
| IR Camera | Generally not available. |
| Privacy Shutter | Usually absent. |
| Encryption Support | Supports BitLocker via Windows. |

## 10. Upgradeability & Repairability

| Category | Evaluation |
| --- | --- |
| RAM Upgrade | Limited; depends on model. |
| Storage Expansion | SSD replacement possible in most models. |
| Battery Replaceability | Internal battery; replaceable by technician. |
| Repair Restrictions | Moderate restrictions. |
| Screw Accessibility | Standard screws, not overly complex. |

## 11. Connectivity & Ports

| Category | Evaluation |
| --- | --- |
| USB Ports | USB-A and USB-C available (no Thunderbolt). |
| Video Output | HDMI supported. |
| Card Reader | SD/microSD available in some models. |
| Ethernet | Available in desktops; rare in laptops. |
| Wi-Fi | Wi-Fi 5 or Wi-Fi 6 in newer models. |
| Bluetooth | Bluetooth 5.0+. |

## 12. Price & Value

| Category | Evaluation |
| --- | --- |
| Price-to-Performance | Good for basic computing tasks. |
| Lifespan Value | Suitable for short- to mid-term use. |
| Configurations | Limited customization options. |
| Upgrade Path | Limited but acceptable for budget systems. |
| Discounts | Often affordable with student pricing. |

## 13. Customer Service & Warranty

| Category | Evaluation |
| --- | --- |
| Warranty Length | Standard 1 year. |
| Repair Type | Depot repair mostly. |
| Support Quality | Average responsiveness. |
| Extended Warranty | Available at additional cost. |
| Accidental Damage | Limited availability. |

## 14. Special Use-Case Evaluation

### For Students

✔ Affordable
✔ Lightweight
✔ Adequate battery
✖ Not powerful

### For Business

✔ Basic security
✖ Not rugged
✖ Limited enterprise features

### For Gaming

✖ No dedicated GPU
✖ Weak cooling

### For Creative Work

✖ Low color accuracy
✖ Weak GPU

## Conclusion

Compaq PCs are best suited for students, home users, and basic office work.
They offer good affordability and acceptable performance, but lack premium build quality, strong graphics, and advanced features.